Washing Dishes

Overview of process

1. Dump any live cultures into bucket with 15% bleach solution.

2. Fully submerge dishes in blue tub containing 15% bleach solution for at least 15 minutes.

3. Rinse dishes with DI water to remove all traces of bleach.

(Remember that bleach + hydrochloric acid = chlorine gas.  That’s very very dangerous).

4. If needed, use a scrub brush to remove any scum on the glassware, and rinse well.

5. Fully submerge dishes in dark blue tub containing 10% hydrochloric acid solution.

6. Rinse dishes three times with DI water and place on drying rack.

7. When dishes are completely dry, transfer to glassware cabinet, checking for cleanliness.

When you’re finished

1. Wipe down the table and sink with a slightly damp sponge. If needed, dry the counter and sink surface with a towel (no standing water should be left).

2. Make sure the lids of the acid and bleach baths are securely closed.

3. Rinse gloves well before putting them on the drying rack.

Reminders

1. DO NOT put dirty dishes in the acid bath. If you contaminate it with other chemicals, you will be responsible for cleaning it out and replacing it.

2. Dishes dry best when slightly tilted (not straight up and down).

3. Acid and bleach are corrosive, and even small droplets of these will damage the sink and counter if not cleaned up.

4. When the bleach bucket for culture disposal is full, it can be dumped down the sink, flushing with plenty of water.  Make sure that the last culture addition was > 30 minutes before dumping.

5. If dishes do not appear clean, do not put them in the glassware cabinet- these need to be washed again.

6. Some of our scrub brushes have metal wires holding the bristles in place.  As you use the brushes, be very careful to not scratch the glassware with the metal part.